OTTO Connect System Specifications

System Specifications

- No base station – Dynamic switching master unit

- Maximum range line-of-sight up to 4000 feet

- Typical range: 1600 to 2000 feet

- Digital Signal Processing

- Spread spectrum – 902-928 MHz frequency

- hopping at 30ms intervals

- Up to 8 speaking users per synchronized group –

- multiple groups supported

- Unlimited listeners

- Talk-time up to 18 hours

Technical Characteristics

- Maximum Output: 500mW

- Frequency-Hopping Spread Spectrum

- Frequency: 900 MHz band (902 – 928 MHz)

- Power

  - Two (2) internal, rechargeable 3000 mAh

- Lithium-Polymer batteries

  - Rechargeable via mini-USB port

Physical Characteristics

- Dimensions (w/o antenna): 5.3” X 3.3” X 1.3”

- Antenna: 3.5”

- Case: Polycarbonate/ABS Blend

- Protective Rubber Boot

- Weight: 15.98 ounces

- Heavy Duty Rotating Belt Clip

- Connectors:

  - Antenna—screw-on type

  - Headset Cable—Hirose locking connector

  - Mini-USB charging port

Environmental Characteristics

- Storage Temperature: -40˚C to +70˚C

- Operating Temperature: -20˚C to +60˚C

- Thermal Shock: -40˚C/+60˚C; 2 hour dwells

- Vibration: General Secured Cargo

- Blowing Rain

- Humidity 24 hour cyclic exposure

- ESD Level 3, Level 4 exposures

FCC ID: ALZMVR900

IC: 10263A-MVR900